Tech/CLW/BCL/03102026

03 Oct 2026

Information restricted

Pages: 20

DRAFT ONLY (TO BE VALIDATED AND APPROVED)

DRAFT TECHNICAL SUBMISSION DOCUMENT

Project

Design, Manufacturing, Supply, Installation, Commissioning and Testing of Smart Locomotive Propulsion Kit for Real Time Condition Monitoring of Traction Motor Bearings on Three Phase Locomotives

Tender Reference Specification Number: CLW/C-D&D/ES/3/0557

Submitted To

Deputy Chief Electrical Engineer (Design and Development)

Design and Development Centre

Chittaranjan Locomotive Works (CLW)

West Bengal, India

TABLE OF CONTENTS

|  | Subject | Page Number |
| --- | --- | --- |
| 1 | Executive Summary |  |
| 2 | System Design Description |  |
| 3 | Bill of Materials (Bill of Materials) Per Locomotive |  |
| 4 | Manufacturing Drawings Index with Engineering Design Rationale and Sketch Notes |  |
| 5 | Type Test Protocol (Type Test Protocol) |  |
| 6 | Data Formats, Railway Management System (RMS) Integration, and Future Approvals |  |
| 7 | Regulatory Compliance Framework and Tender Requirement Mapping |  |
| 8 | Declaration |  |

EXECUTIVE SUMMARY

This document serves as a proposal for the design, manufacturing, supply, installation, commissioning and testing of a Smart Locomotive Propulsion Kit. The system is engineered to provide real time condition monitoring of Traction Motor (TM) bearings on three phase locomotives operating across the Indian Railways (IR) network.

Each locomotive has two bogies and each bogie has three traction motors, installed with bearings on the Drive End (DE) and Non Drive End (NDE), totalling twelve bearing monitoring points per locomotive. The proposed system is an Internet of Things (IoT)-based, non intrusive predictive maintenance solution that monitors vibration and temperature signatures from all twelve TM bearings in real time, enabling early fault detection at least six months before bearing replacement is required.

The design strictly adheres to all operating and service conditions specified in the tender document CLW/C-D&D/ES/3/0557, including ambient temperatures from minus 30 degrees Celsius to plus 55 degrees Celsius, train speeds up to 160 kilometres per hour, relative humidity up to 98 percent, altitudes up to 1700 metres above sea level, and extreme dust concentrations up to 16 milligrams per cubic metre. All hardware complies with the mandatory Research Designs and Standards Organisation (RDSO) specifications and International Electro technical Commission (IEC) standards referenced in the tender.

This proposal explicitly details the hardware specifications, signal transmission strategies, power supply architecture, data aggregation methods, real time streaming protocols, fault threshold limits, software architecture, database schema, dashboard wireframe, process flow and the complete regulatory compliance framework.

Every technical parameter is defined in full, every requirement from the tender document is explicitly mapped to its governing RDSO code and our proposed engineering solution, leaving no room for ambiguity or unanswered questions during the review and approval process.

VISUAL LAYOUT OF THE 12 MONITORING POINTS

LOCOMOTIVE BODY

[ FRONT BOGIE]                                         [ REAR BOGIE ]    = TOTAL BOGIES 2

(3 Axles, 3 Motors)                                       (3 Axles, 3 Motors)

| AXEL + TRACTION Motor( TM) 1: [DE (DRIVE END) Bearing #1] + [NDE (NON DRIVE END) Bearing #2] | AXEL+ TM 4: [DE Bearing #7] + [NDE Bearing #8] |
| --- | --- |
| AXEL + TM 2: [DE Bearing #3] + [NDE Bearing #4] | AXEL+ TM 5: [DE Bearing #9] + [NDE Bearing #10] |
| AXEL+ TM 3: [DE Bearing #5] + [NDE Bearing #6] | AXEL + TM 6: [DE Bearing #11] + [NDE Bearing #12] |

- TOTAL FRONT BOGIE: 6 Monitoring Points

- TOTAL REAR BOGIE:  6 Monitoring Points

- GRAND TOTAL:       12 Monitoring Points per Locomotive

- PROBLEM STATEMENT: AUTOMATION: HW + SW FOR 12 individual Vibration cum Temperature Sensor Units

ILLUSTRATION ONLY: SINGLE TRACTION MOTOR CROSS SECTION (TM-1)

TRACTION MOTOR STATOR (STATIONARY HOUSING)

ROTATING SHAFT

NON DRIVE END

BEARING LOCATION

DRIVE END

BEARING LOCATION

SENSOR #2

VIBRATION + TEMP

SENSOR # 1

VIBRATION + TEMP

(IP 69 K)

END COVER

GEAR CASE

PINION + AXEL

- DE Sensor (#1): Mounted on gear side end cover using non intrusive SS316L bracket

- NDE Sensor (#2): Mounted on opposite end cover using non intrusive SS316L bracket

- Both sensors measure vibration (2 Hz - 10000 Hz) + temperature (MINUS 50 DEG C to PLUS 200 DEG C)

- No drilling AND / OR or welding required - uses existing cap screw holes

| COMPLETE SENSOR NODE MAPPING - ALL 12 BEARINGS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sensor Node ID | Bogie | Traction Motor ™ | Axle Position | Bearing Location | Measurement Type |
| #1 | FRONT BOGIE (BOGIE 1) | TM 1 | 1 | DRIVE END (DE) | VIBRATION + TEMP |
| #2 | FRONT BOGIE (BOGIE 1) | 1 | 1 | NON DRIVE END (NDE) | VIBRATION + TEMP |
| #3 | FRONT BOGIE (BOGIE 1) | TM 2 | 2 | DRIVE END (DE) | VIBRATION + TEMP |
| #4 | FRONT BOGIE (BOGIE 1) | 2 | 2 | NON DRIVE END (NDE) | VIBRATION + TEMP |
| #5 | FRONT BOGIE (BOGIE 1) | 3 | 3 | DRIVE END (DE) | VIBRATION + TEMP |
| #6 | FRONT BOGIE (BOGIE 1) | 3 | 3 | NON DRIVE END (NDE) | VIBRATION + TEMP |
| #7 | REAR BOGIE (BOGIE 2 | TM 4 | 4 | DRIVE END (DE) | VIBRATION + TEMP |
| #8 | REAR BOGIE (BOGIE 3 | TM 4 | 4 | NON DRIVE END (NDE) | VIBRATION + TEMP |
| #9 | REAR BOGIE (BOGIE 4 | 5 | 5 | DRIVE END (DE) | VIBRATION + TEMP |
| #10 | REAR BOGIE (BOGIE 5 | 5 | 5 | NON DRIVE END (NDE) | VIBRATION + TEMP |
| #11 | REAR BOGIE (BOGIE 6 | TM 6 | 6 | DRIVE END (DE) | VIBRATION + TEMP |
| #12 | REAR BOGIE (BOGIE 7 | TM 6 | 6 | NON DRIVE END (NDE) | VIBRATION + TEMP |

ENGINEERING NOTES:

- Non Intrusive Mounting: All 12 sensor nodes are mounted using custom SS316L brackets that utilize existing end-cover cap screw holes. No drilling, welding, or structural modification to the traction motor housing is required.

- Data Concentrator Integration: The single Data Concentrator mounted on the locomotive body aggregates data from all 12 sensor nodes via wired RS485/Modbus or 4-20mA current loop connections, then transmits to the cloud via 4G/5G cellular network.

- Bogie Drop Clearance: Sensor protrusion is kept below 40mm from the end cover face to ensure no interference during bogie removal for maintenance.

- Universal Design: The same sensor hardware (Nodes #1-#12) is used for all 12 bearing locations. Only the mounting bracket geometry varies slightly between DE and NDE positions.

- Redundancy: Each sensor node contains duplex Pt100 temperature sensors for redundancy in case one sensing circuit fails under bogie stress.

SENSOR NODE DISTRIBUTION

LOCOMOTIVE SIDE ELEVATION

REAR CAB

FRONT CAB

LCD Display

Node Alerts)

[DATA CONCENTRATOR / EDGE GATEWAY]

MTD ON LOCO BODY/ UNDERFRAME                                                   Aggregates data from all 12 Sensor Nodes

BOGIE 2

BOGIE 1

TM 2

#3 (DE)

#4

(NDE)

TM 1

#1 (DE

#2 NDE

TM 3

#5(DE)

#6 (NDE)

TM 4

#7 (DE

#8 NDE

TM 5

#9 (DE)

#10

(NDE)

TM 7

#11

(DE)

#12 (NDE)

WHEEL

WHEEL

SENSOR NODE LEGEND

- #1, #2 – TM1 (DE (DRIVE END – GEAR CASE SIDE), NDE (OTHER SIDE) FRONT BOGIE AXEL 1

- #3,#4 – TRACTION MOTOR (DE) & NDE)- FRONT BOGIE , AXEL 2

- #5,#6 -   TRACTION MOTOR (DE) & NDE)- FRONT BOGIE , AXEL 3

- #7,#8 -     TRACTION MOTOR (DE) & NDE)- FRONT BOGIE , AXEL 4

- #9,#10 - TRACTION MOTOR (DE) & NDE)- FRONT BOGIE , AXEL 5

- #11,#12 - TRACTION MOTOR (DE) & NDE)- FRONT BOGIE , AXEL 6

- SYSTEM DESIGN DESCRIPTION

2.1. Hardware Architecture

2.1.1. Sensor Nodes (12 per Locomotive)

Design

Non intrusive, surface mounted modular units utilizing custom Stainless Steel 316L (SS316L) clamping brackets that do not interfere with TM end caps or bogie drops. No drilling, welding or machining of the TM casing is required. The brackets utilize existing end cover cap screws, replacing them with slightly longer high tensile SS316 bolts that pass through the adapter ring, clamping it flush against the TM end cover face.

Sensing Elements

Temperature

- Integrated Duplex Platinum 100 Ohm (Pt100) Resistance Temperature Detector (RTD), Class A precision, conforming to International Electrotechnical Commission (IEC) standard 60751.

- The duplex configuration provides two separate internal sensing circuits for critical redundancy. Temperature range: minus 50 degrees Celsius to plus 200 degrees Celsius.

Vibration

Integrated Electronics Piezo Electric (IEPE) Accelerometer with a rugged crystal framework designed for heavy shocks.

- Frequency range: 2 Hertz to 10,000 Hertz.

- Measurement range: plus or minus 16 standard gravity (g).

- Sensitivity: 100 millivolts per standard gravity (mV/g).

- Shock survival: 500 g to 5000 g peak.

Edge Processing

- Localized micro processing performs initial Fast Fourier Transform (FFT) and data filtering to minimize payload over the cellular network.

Sampling data rate: at least 2000 Hertz (Hz).

Housing Material

- Marine grade SS316L Stainless Steel to survive rock strikes from track ballast and corrosion from acidic rail cleaning chemicals.

Ingress Protection (IP) Rating

- \IP69K, certifying the hardware can withstand close range, high pressure water jets at temperatures up to 80 degrees Celsius during routine depot washdowns.

Connectors: Rugged, threaded Metric 12-millimetre (M12) metal circular connectors featuring fluoroelastomer O-rings to maintain the IP69K seal under pressure. NO Plastic connectors

2.1.2. Data Concentrator (Data Aggregation Hub)

- Design: Centralized edge computing unit housed in a die cast Aluminum Alloy (EN AB-46000) IP66/IP67 ruggedized enclosure, mounted securely on the locomotive body or underframe to ensure it remains undisturbed during bogie maintenance.

- Connectivity: Integrated Global System for Mobile Communications (GSM) router supporting 2G/3G/4G/5G with auto switching mode (micro Subscriber Identity Module (SIM) or embedded SIM (eSIM)), Wireless Fidelity (Wi-Fi), Bluetooth (BT), Modbus Recommended Standard 485 (RS485) and Modbus Transmission Control Protocol/Internet Protocol (TCP/IP) interfaces.

- Buffering: High capacity local Non Volatile Memory Express (NVMe) solid state buffer to store timestamped, geotagged data during network blackouts, ensuring zero data loss. Data is held until cellular network coverage becomes available and then transmitted at the earliest.

- Thermal Management: Passive conduction cooling via extruded aluminium fins and Thermal Interface Material (TIM) pads. No active cooling fans are used, maintaining the IP67 seal.

2.1.3. Power Supply and Protection

- Input: 110 Volts Direct Current (V DC) nominal supply from the locomotive battery bus.

- Converters: Isolated Direct Current to Direct Current (DC-DC) converters stepping down the 110V DC supply to system operating voltages (24V DC for sensors, 12V/5V DC for edge processing). Input acceptance range: 66V to 154V DC to handle transient dips and surges.

- Galvanic Isolation: Minimum 1.5 kilovolts (kV) to 3 kV Alternating Current (AC) isolation between the 110V DC train battery bus and internal low voltage electronics.

- Holdup Time: 10 to 20 milliseconds (ms) using integrated capacitor banks to keep the edge gateway alive during brief power supply dropouts.

- Protection: Dedicated railway grade Miniature Circuit Breakers (MCBs) and Transient Voltage Suppression (TVS) diode arrays clamping load-dump transients to 140V.

2.1.4. Cab Displays

- Two (2) ruggedized Liquid Crystal Display - Thin Film Transistor (LCD-TFT) displays (one per cab), sunlight readable (greater than 1000 nits brightness), IP65 front panel, providing real time actionable alerts and colour coded health indices to the Loco Pilot. Powered from the 110V DC source via a suitable power supply unit designed by the supplier.

2.2. Signal Transmission Strategy

- To transport signals through the intense electromagnetic fields of a traction motor without data corruption, analog cabling is converted to robust, noise immune outputs as close to the bearing housing as possible.

Option A: 4-20 Milliamperes (mA) Current Loop (Analog Solution)

- Unlike voltage signals (0-10V) which degrade over long wire runs and pick up motor hum, a current loop maintains absolute signal integrity. The current remains constant regardless of wire length or electromagnetic induction.

- The temperature transmitter converts the Pt100 RTD resistance variation into a proportional 4-20mA signal. A broken cable drops the current to 0mA, providing instant automated open circuit error detection.

Option B: Modbus over RS485 (Digital Solution)

- Uses differential signalling (two wires carrying opposite voltages). When electromagnetic noise strikes the cable, it affects both wires equally. The receiving node calculates the difference, completely cancelling out shared noise.

- Excellent for daisy chaining multiple sensors back to a single bogie junction box using a single twisted pair cable.

2.3. Cable and Conduit Integrity

- Cable Insulation: Teflon insulated internal conductors wrapped in an integrated braided stainless steel shield.

- Mechanical Armour: Entire cable run enclosed in heavy duty, liquid tight, flexible polyamide or braided steel conduit with generous drip loops for bogie suspension displacement.

- Fire Protection: All cables protected by fire retardant shrinkable sleeve throughout routing, conforming to European Norm (EN) 45545-2 Hazard Level 3 (HL3) and EN 60332-3-25.

2.4. Software and Artificial Intelligence (AI) Analytics Architecture

2.4.1. Type of Platform

- Cloud Infrastructure: Hosted on a scalable, non-proprietary cloud computing platform (such as Amazon Web Services (AWS) or Microsoft Azure India Region) with geo redundant mirrored servers at each geographical location within India, complying with national data localization laws.

- Edge Computing Platform: The Data Concentrator runs a Linux based Real Time Operating System (RTOS) on an industrial grade System on Chip (SoC) processor (such as NXP i.MX8 or equivalent), capable of performing on device FFT, health index calculation and anomaly detection.

- Communication Protocols (Cloud): Message Queuing Telemetry Transport (MQTT) over encrypted Hypertext Transfer Protocol Secure (HTTPS) for data ingestion. Representational State Transfer (REST) Application Programming Interface (API) for data access.

- Communication Protocols (On board): Modbus TCP/IP and RS485 for integration with on board Railway Management System (RMS) and Train Control and Management System (TCMS).

2.4.2. Internet of Things (IoT) Devices Used

| Device | Type | Quantity per Locomotive | Function |
| --- | --- | --- | --- |
| Vibration cum Temperature Sensor Unit | IEPE (Integrated Electronics Piezo Electric) Accelerometer + Duplex Pt100 RTD | 12 | Captures vibration (2 Hz to 10,000 Hz) & temperature (minus 50 to plus 200 degrees Celsius) from each TM bearing |
| Data Concentrator | Industrial Edge Gateway with 4G/5G Router | 1 | Aggregates, processes, buffers and transmits all sensor data |
| Speed Sensor Interface | Opto isolated signal conditioner | 1 | Interfaces with existing locomotive speed sensor for speed gating |
| LCD-TFT Cab Display | Ruggedized touchscreen monitor | 2 | Displays real time alerts to Loco Pilot |
| DC-DC Power Converter | Isolated railway-grade power module | 2 | Converts 110V DC to 24V/12V/5V DC |

2.4.3. Code and Programming Languages Used

| Component | Programming Language / Framework | Purpose |
| --- | --- | --- |
| Sensor Node Firmware | C / C++ | Real time data acquisition, FFT computation, local filtering |
| Data Concentrator Edge Software | Python 3.x with C++ acceleration modules | Data aggregation, AI inference, MQTT publishing, local buffering |
| AI Diagnostic Engine | Python (TensorFlow Lite / PyTorch Mobile) | Bearing defect classification (BPFO, BPFI, BSF, FTF detection) |
| Cloud Backend | Python (Django / FastAPI) with PostgreSQL + InfluxDB | Data ingestion, REST API, time series storage |
| Web Dashboard Frontend | JavaScript (React.js / Angular) with D3.js for charts | Fleet visualization, bogie schematics, trend analysis |
| Mobile Application | Flutter (Dart) for cross platform (iOS / Android) | On the go fleet monitoring for maintenance staff |
| Database Query Language | Structured Query Language (SQL) for relational data; Flux for time series | Data retrieval and analytics |

2.4.4. Database Schema

Relational Tables (Asset and Configuration Management)

| Table Name | Primary Key (PK) / Foreign Key (FK) | Columns and Data Types | Description |
| --- | --- | --- | --- |
| Loco_Master Asset tracking • Acts as the primary registry for the fleet. Every physical train engine has exactly one entry here. | PK: Loco_ID (Unique iden for each loco) The unique identification number of the locomotive ( Loco #31024). | Loco_Class (Variable Character) The model or type of train ( WAP-7, WAG-9). This is crucial because different train types behave differently under stress. Zone (Variable Character) + (The regional division and physical workshop responsible for maintaining this specific locomotive) Commission Date (Date) - The date the train was put into active service, helping engineers track its overall age. | Stores static details of the locomotive and its home shed |
| TM_Bearing_Asset Component & IoT Sensor Mapping Acts as the primary registry for the fleet. Every physical train engine has exactly one entry here. | PK: Node_ID, FK: Loco_ID PK: Unique identifier for each specific bearing location/sensor node. FK: Links this specific bearing back to its parent locomotive in | Bogie_No (Integer)+ TM_Position (Integer 1-6) The exact physical location of the motor (e.g., Bogie 1, Motor 3). End_Type (Enumeration: DE, NDE), (Specifies if the bearing is on the Drive End (DE) (gear side) or Non-Drive End (NDE) (opposite side). Sensor_MAC (Variable Character), Install_Date (Date) (The unique hardware address of the wireless sensor attached to that bearing. When data streams in, the system uses this MAC address to know which bearing is transmitting) | Maps each of the 12 TM bearings to their physical location and sensor hardware |
| Alert_Thresholds (AI & Rules Configuration) Stores the rules that the predictive maintenance AI uses to trigger alarms. Instead of hardcoding hard limits for every train, the Chief Mechanical Engineer (CME) can configure different vibration or temperature thresholds based on the Loco_Class. | PK: Config_ID Unique identifier for the configuration rule. | Loco_Class (Variable Character) Dictates which train model these rules apply to (e.g., heavy freight trains will have higher normal limits than high-speed passenger trains). Normal_Limit (Float) Minor_Limit (Float), Intervention_Limit (Float), Immediate_Limit (Float) A graduated scale of safety limits. If sensor data crosses these float (decimal) values, the system escalates the alert level accordingly ----- Normal Minor (Watch closely) Intervention (Schedule maintenance) Immediate (Stop the train safely to prevent catastrophic failure). | Stores configurable AI threshold limits set by the Chief Mechanical Engineer (CME) How the Tables Connect (Relational Workflow) Loco Master (1) TM Bearing Asset (Many) (Triggered Data) ------ Predictive Maintenance AI Controlled and Evaluates against set rules (Alert Thresholds) |

Time Series Tables (High Frequency Telemetry & Analytics)

| Table Name | Partition Key | Columns and Data Types | Description |
| --- | --- | --- | --- |
| Telemetry_Raw_Buffer | Timestamp | Node_ID (Variable Character), Speed_kmph (Float), Temp_C (Float), Vibration_RMS (Float), Raw_Waveform (Binary Large Object) | Stores raw unfiltered data temporarily, retained for 48 hours unless anomaly detected |
| Telemetry_Aggregated | Timestamp | Node_ID (Variable Character), Speed_kmph (Float), Temp_C (Float), RMS_Velocity (Float), Peak_BPFO (Float), Peak_BPFI (Float), Peak_BSF (Float), Peak_FTF (Float) | Stores edge-processed FFT data, captured only when Speed greater than 10 kilometres per hour |
| Health_Index_Log | Timestamp | Node_ID (Variable Character), Health_Score (Integer 0-100), Health_State (Enumeration), Fault_Detected (Variable Character), Confidence_Percent (Float) | Logs final AI-calculated health index and specific defect diagnosis |

2.4.5. Dashboard Wireframe

SMART LOCOMOTIVE PROPULSION KIT - FLEET MANAGEMENT DASHBOARD [USER: CME/HQ]  [NOTIFICATIONS: 3]

NAVIG

- FLEET OVERVIEW

- LOCO DETAIL

- ANALYTICS

- SETTING & CONFIG

MAIN DASHBOARD VIEW (FLEET OVERVIEW)

GEOSPATIAL MAP VIEW

LIVE TR

- ICON:  LOCO 100 - RED

- ICON: LOCO 200 - YELOW

- ICON: LOCO 300 - GREEN

FLEET HEALTH SUM (CHRAT)

- NORMAL

- MINOR

- INTERVENTION

- IMDT REPLACMENT

BOGIE SCHMATIC VIEW (SELCET WAG -9 LOCO NUMBER 300)

BOGIE 1                            					BOGIE 2

TM1 DE – GREEN   OK         TM1 NDE -  GREEN OK

TM2 DE – GREEN OK           TM2 NED – GREEN OK

VIBRATION & TREND ANALYSISI – TN2 NDE

Y AXIS (VIBRATION (MM/S)

X AXIS (DIST/ TIME (30 DAYS)

ACTIONABLE ALERTS AND AI DIGONISISI

- CRTICIAL ALERTS (RED) – LOCO 100 TM2 – NDE

  - FAULT – OUTER RACE DEFECT (BPFO PEAK DETECTED)

  - HLTH INDEX – 12/100 (IMDT REPLACMENT)

  - ACTION: PLAN REPLACMENT AT NXT SCHEDULE PERIODIC OVERALL

  - NOTFIY SHED

[ACK]                             [VIEW RAW DATA]              [DOWNLOAD PDF] / MAIL

2.4.6. End to End Process Flow Diagram

PHYSICAL LAYER: TRACTION MOTOR ASSEMBLY]

(Mechanical Vibration & Thermal Energy)

TM BEARING (DE / NDE)

[EDGE LAYER 1: SENSOR NODE (Mounted on TM)]

- Pt100 reads Temperature continuously.

- IEPE Accelerometer reads Vibration (2 Hz to 10,000 Hz).

- Microcontroller performs local Fast Fourier Transform (FFT).

SENSOR NODE EDGE CPU

[CAL RMS, RMS]

SPEED GATING LOGIC

IF TRAIN LESS THAN 10KM/H -----SUSPEND VIBRATION CAPTURE

IF TRAIN G/THAN 10KM/HR ----- CAPTURE + PROCESS DATA

(Wired RS485 / 4-20mA Current Loop - Encrypted Payload)

[EDGE LAYER 2: DATA CONCENTRATOR (Mounted on Loco Body)]

- Aggregates data from all 12 Sensor Nodes.

- Tags data with Global Positioning System (GPS) Location, Date, Time, Loco ID.

- Checks Cellular Network (4G/5G) availability.

DATA CONCENTRATOR CPU

DATA AGGREGATION

LOCAL NVME BUFFERING

NW BLACKOUT LOGIC

IF NW NOT AVALIABLE ----- WRITE DATA ON LOCAL MvME

IF NW AVALIABLE ----- TRANSMIT DATA IMDLTY

(Encrypted MQTT / HTTPS via 2G/3G/4G/5G Cellular Network)

[CLOUD LAYER: CENTRAL SERVER AND AI ANALYTICS]

Decrypts data, validates payload, routes to databases.

CLOUD INGESTION GATEWAY

Analyses FFT peaks against characteristic frequencies

- Ball Pass Frequency Outer Race (BPFO) ---Outer Race Defect

- BALL PASS FREQ INNER RACE (BPFI)  --- INNER RACE DEFECT

- BALL SPIN FREQUENCY (BSF)   --- ROLLER DEFECT

- FUNDAMENTAL TRAIN FREQUENCY (FTF)  ---- CAGE DEEFECT

COMPARE AI OUTPUT AGIANST CONFIGURED THRESHOLDS

TIER/ COLOUR

- TIER 1 : NORMAL – GREEN

- 2: MINOR – YELLOW

- 3: 3 / 4 MONTHS – ORANGE

- 4: IMDT - RED

AI DIAGONISTIC ENGINE

ML LEARNING ALGO

HLTH INDEX CLACULATOR

[PRESENTATION AND ACTION LAYER]

AUTOMATED NOTFN

EMAIL/ WHATTS APP/ SMS

SEND TO SHED MASTER, MAINT ENGR, DIV MGR

ON BOARD LCD DISPLAY

LOCO DRIVER CABIN

- REAL TIME HLTH

- IMDT ALERTS

- ACTIONABLE PROMPTS

WEB DASH BOARD

(FOR CME/ HQ/ SSE

- FLEET GEO MAP

- BOGIE SCHMATICS

- VIBRATION TRENDS

- THERMAL TRENDS

- RAW DATA EXTRACTION

2.4.7. Railway Fault Threshold Limits

Temperature Thresholds (Pt100 RTD):

- Normal Operating Range: 40 degrees Celsius to 85 degrees Celsius under normal load and track conditions.

- Warning Limit (Amber Alert): 95 degrees Celsius, OR a rate of temperature rise exceeding 2 degrees Celsius per minute. Action: Flag for inspection at the next scheduled depot rotation.

- Critical Limit (Red Alert): 105 degrees Celsius to 110 degrees Celsius absolute. Action: Immediate driver notification to limit speed or schedule emergency train removal.

Vibration Thresholds (IEPE Accelerometer)

- Overall Vibration Velocity (10 Hz to 1,000 Hz):

          - Warning: 4.5 millimetres per second Root Mean Square (mm/s RMS).

          - Critical: 7.1 mm/s RMS.

- High Frequency Acceleration (1 kHz to 10 kHz):

          - Warning: 3.0 standard gravity (g) Peak.

          - Critical: 7.0 g Peak.

BILL OF MATERIALS (BOM) - PER LOCOMOTIVE

|  | Item Description | Specifications | UPM | Quantity |
| --- | --- | --- | --- | --- |
| 1 | Vibration cum Temp Sensor Unit | IP69K, SS316L, Duplex Pt100 RTD + IEPE Accelerometer, 2 Hz to 10 kHz, EN 50155 / EN 61373 Category 3 | NUMERS | 12 |
| 2 | Data Concentrator / Data Aggregation Hub | IP66/67, 4G/5G, Wi-Fi, Bluetooth, RS485, Edge Buffer, EN 50155 / EN 61373 Category 2 | NUMERS | 1 |
| 3 | Speed Sensor Interface Module | Opto-isolated interface to existing locomotive speed sensor | SET | 1 |
| 4 | Power Supply and MCB Panel | 110V DC to 24V/12V/5V DC Isolated Converters, Railway Grade MCBs, TVS Surge Protection | SET | 1 |
| 5 | LCD-TFT Cab Display | Ruggedized, Sunlight readable (greater than 1000 nits), 110V DC compatible, IP65 front panel | NUMERS | 2 |
| 6 | Mounting Brackets and Hardware | SS316L Custom non intrusive clamps, anti-vibration nuts and bolts | SET | 12 |
| 7 | Communication and Power Cables | FEP/PTFE insulated, braided SS shield, Fire Retardant (EN 60332-3-25), EN 45545-2 HL3 | LS | Lump Sum |
| 8 | Cable Protection and Routing | Fire retardant shrinkable sleeves, braided sleeves, conduits, glands (IP67) | LS | Lump Sum |
| 9 | Cable Cleats and Ties | Ultraviolet and High Temperature resistant, SS304 ties where applicable | LS | Lump Sum |
| 10 | M12 Metal Circular Connectors | Threaded, Viton O-rings, IP69K rated | AS REQD | As Required |
| 11 | Cloud Hosting and Cellular Data Plan | 3 Yr 4G/5G Data Service, Cloud Server Hosting, AI Analytics License | LOT | 1 |
| 12 | Software Dashboard Access | Web based fleet management dashboard (Lifetime free access for IR) | LOT | 1 |

4. MANUFACTURING DRAWINGS INDEX WITH ENGINEERING DESIGN RATIONALE AND SKETCH NOTES – TO BE ATTACHED BEFORE SUBMISSION

4.1. DWG 01: General Assembly of Sensor Unit and Non Intrusive Mounting Bracket on TM Bearing Housing (DE + NDE Views)

4.2. DWG 02: Data Concentrator Enclosure Mechanical Design, Thermal Dissipation Layout & Internal Printed Circuit Board (PCB) Stack up

4.3. DWG 03: Loco Level Wiring Schematic & Cable Routing Diagram

4.4. DWG 04: 110V DC Power Distribution Panel, MCB Sizing & Isolation Circuit Diagram

4.5. DWG 05: Cab LCD Display Mounting Arrangement & Interfacing with Concentrator

4.6. DWG 06: System Communication Topology (Sensor to Concentrator & Concentrator to Cloud/RMS)

5. TYPE TEST PROTOCOL (TTP) (TO BE VLAIDATED)

All components and the integrated system will undergo rigorous type testing at National Accreditation Board for Testing and Calibration Laboratories (NABL)

| Test Category | Test Description | Applicable Standard | Pass Criteria |
| --- | --- | --- | --- |
| Environmental | Temperature Cycling and Damp Heat | EN 50155:2017 | Operates flawlessly from minus 40 degrees Celsius to plus 125 degrees Celsius (Sensor) & minus 25 degrees Celsius to plus 70 degrees Celsius (Concentrator) |
| Mechanical | Shock and Vibration Testing | EN 61373:2010 | Sensors: Category 3 (Axle/Bogie mounted). Concentrator: Category 2 (Car body mounted). No mechanical degradation or signal loss |
| Electromagnetic | Electromagnetic Compatibility (EMC) Emission and Immunity | EN 50121-3-2 | No interference with loco control systems or passenger Wi-Fi/Cellular networks |
| Ingress Protection | Dust and Water Ingress | IEC 60529 | IP69K for Sensors; IP66/67 for Data Concentrator |
| Fire and Safety | Flammability, Smoke Density, Toxicity | EN 45545-2 | Compliance with Hazard Level 3 (HL3) for all cables, sleeves, and non metallic enclosures |
| Electrical | Insulation Resistance and Dielectric Withstand | EN 50155 | Greater than 100 Mega-ohms at 500V DC. Withstands 110V DC loco supply fluctuations |
| Functional | Network Blackout Buffer Test | CLW Specification | Data successfully cached during simulated 24 H NW loss and batch uploaded upon reconnection without corruption |
| Functional | Latency and Speed Gating Test | CLW Specification | Vibration data capture strictly gated above 10 kmph. Live transmission latency less than 60 seconds |
| Functional | Bearing Defect Detection Test | CLW Specification | System detects simulated BPFO, BPFI, BSF, FTF signatures at least 6 months before replacement threshold |

6. DATA FORMATS, RAILWAY MANAGEMENT SYSTEM (RMS) INTEGRATION, AND FUTURE APPROVALS

6.1. Open Communication Architecture

- Onboard Interfacing: The Data Concentrator exposes an open Modbus TCP/IP & RS485 register map. This allows the locomotive native RMS or Train Logging Computer (TLC) systems to pull real time alert statuses directly via standard industrial protocols.

- Cloud to Server API: A fully documented REST API with JavaScript Object Notation (JSON) payloads will be provided to IR/CLW developers. This includes complete mapping of geospatial data, timestamping &  raw/processed vibration metrics.

- Raw Data Access: Zonal Railways and CLW will have full, unrestricted access to extract raw and processed data in standard usable formats including Comma Separated Values (CSV), JSON & Structured Query Language (SQL).

6.2. Commitment to Future CLW Approvals

- Any future modifications to User Interface (UI), dashboard data visualization or data formatting required for integration with newer onboard RMS architectures introduced by Indian Railways will be developed and submitted to CLW for approval.

- Firmware updates to the Concentrator edge computing module to support new open standard protocols or RMS handshakes will be provided Ove The Air (OTA) at no additional cost during the warranty and Annual Maintenance Contract (AMC) periods.

- All proprietary communication protocols or data formats used by the system will be shared with CLW as mandated in the tender specification.

7. REGULATORY COMPLIANCE FRAMEWORK AND TENDER REQUIREMENT MAPPING

This section explicitly maps every factor, piece of equipment, and standard considered in this proposal against both the tender document requirements (CLW/C-D&D/ES/3/0557) and the governing RDSO specifications.

7.1. Comprehensive Compliance Matrix (TO BE VLAIDATED)

|  | Equipment / Factor | RDSO Specification Number | International Standard | Proposed Solution |
| --- | --- | --- | --- | --- |
| 1 | Sensors, Brackets, Concentrator, Cables | RDSO/2014/EL/SPEC/0114 and RDSO/2010/EL/SPEC/0108 (Revision 2) | IEC 61373 (Category 1 Class B / Category 2 / Category 3) | All bogie mounted sensors tested to Category 3 (500g shock, 5000g survival). Concentrator tested to Category 2. Mounting brackets use anti vibration SS316 hardware |
| 2 | All Electronic Components | ELRS/SPEC/SI/0015 (latest revision) | EN 50155:2017 (Class TX) | Sensors rated minus 40 to plus 125 degrees Celsius. Concentrator rated minus 25 to plus 70 degrees Celsius. Passive conduction cooling eliminates fan failure risk |
| 3 | Speed Sensor, Data Gating Logic | RDSO/2016/EL/SPEC/0123 (Revision 0) | IEC 60751 | Speed gating activates vibration capture above 10 kmph. Speed signal interfaced from existing locomotive speed sensor via opto-isolated module |
| 4 | Sensor Housing, Concentrator Enclosure, Connectors | RDSO/2008/CG-04 (Revision 05) | IEC 60529 | IP69K rating on all bogie-mounted sensors and connectors. Hermetic sealing withstands high-pressure washdowns and extreme dust ingress |
| 5 | PCBs, Enclosures, Connectors | RDSO/2008/CG-04 (Revision 05) | EN 50155 | Conformal coating on all PCBs. SS316L marine-grade housing. Viton O-rings on M12 connectors. Die-cast aluminum enclosure with powder coating |
| 6 | Data Concentrator, Cloud AI Engine | RDSO/2016/EL/SPEC/0123 (Revision 0) | N/A (Proprietary AI) | Edge AI detects BPFO, BPFI, BSF, FTF signatures. NVMe buffer stores data during network blackout. Batch upload on reconnection |
| 7 | Data Concentrator GSM Module | RDSO/SPN/196/2020 (Version 4.0) | EN 50121-3-2 | Integrated 4G/5G router with auto-fallback to 2G/3G. Micro SIM and eSIM support. 3-year data plan included in supply scope |
| 8 | Sensor Unit, AI Algorithms | RDSO/2016/EL/SPEC/0123 (Revision 0) | ISO 20816-21 | Hardware and software universal to all bearing types. AI algorithms configurable for any bearing geometry via dashboard |
| 9 | Data Concentrator, AI Engine | RDSO/2016/EL/SPEC/0123 (Revision 0) | DIN 55667 | Single Data Concentrator per loco. Trend analysis with 4 tier colour coded health index provides 6 month advance warning |
| 10 | LCD-TFT Cab Display, Concentrator Software | RDSO/2016/EL/SPEC/0123 (Revision 0) | N/A | Two LCD-TFT displays (one per cab) show real time colour coded alerts. Thresholds configurable by operator via dashboard |
| 11 | Data Concentrator I/O | RDSO/SPN/196/2020 (Version 4.0) | IEC 62236-3-1 | Modbus TCP/IP and RS485 open register map. REST API with JSON payloads. All protocols shared with CLW |
| 12 | Vibration Sensor Unit | RDSO/2016/EL/SPEC/0123 (Revision 0) | IEC 61373 Cat 3 | IEPE Accelerometer: 2 Hz to 10,000 Hz (exceeds 5000 Hz requirement), plus/minus 16g, 100 mV/g, 2000 Hz sampling rate |
| 13 | Temperature Sensor Unit | RDSO/MP.0.2400.64 (latest version) | IEC 60751 | Duplex Pt100 RTD, Class A precision, minus 50 to plus 200 degrees Celsius (exceeds requirement for bearing specific monitoring) |
| 14 | Data Concentrator | RDSO/PE/SPEC/EMU/0196-2019 (Revision 0) and RDSO/2008/CG-04 (Revision 05) | EN 50155, EN 50121-3-2 | IP66/67 enclosure, 110V DC input (66-154V range), 4G/5G with auto witching, Modbus RS485 and TCP/IP |
| 15 | All Power and Communication Cables | ELRS/SPEC/ELC/0019 (Revision 4 | dated 22 February 2018) | EN 60332-3-25, EN 45545-2 FEP/PTFE insulated, braided SS shield, fire retardant shrinkable sleeve throughout routing, EN 45545-2 HL3 compliant |
| 16 | Speed Sensor Interface | RDSO/2010/EL/SPEC/0108 (Revision 2) | EN 50155, EN 50121-3-2, EN 45545-2 | Opto-isolated interface to existing locomotive speed sensor. No additional sensor required unless existing unit is incompatible |
| 17 | All Components | ELRS/SPEC/SI/0015 (latest revision) | EN 50155 | All components rated for greater than 5 years service life. Defective parts replaced within 3-year warranty at no cost |
| 18 | AI Diagnostic Engine, Dashboard | RDSO/2016/EL/SPEC/0123 (Revision 0) | ISO 20816-21, DIN 55667 | 4 tier colour coded health index: GREEN (Normal), YELLOW (Minor), ORANGE (3 / 4 Months), RED (Immediate). AI detects inner ring, outer ring, roller, cage defects |
| 19 | Installation Process | RDSO/2016/EL/SPEC/0123 (Revision 0) | N/A | Non intrusive mounting (no drilling/welding) enables complete installation within 5 days. Supplier deputes engineers during installation |
| 21 | Data Concentrator Firmware | RDSO/2016/EL/SPEC/0123 (Revision 0) | N/A | GPS module integrated in concentrator. Every data packet tagged with GPS coordinates, date, and time before transmission |

8. DECLARATION

We hereby submit this Final Technical Proposal, including the complete System Design, Bill of Materials, Manufacturing Drawings Index with Engineering Design Rationale, Type Test Protocol, Data Formats and RMS Integration Plan, the comprehensive Regulatory Compliance Framework mapped against both the tender document requirements (CLW/C-D&D/ES/3/0557) and all applicable Research Designs and Standards Organisation (RDSO) specifications.

We confirm the following:

- The proposed design strictly complies with all operating conditions, space constraints & functional requirements outlined in Tender Specification Number CLW/C-D&D/ES/3/0557.

- All hardware components meet or exceed the mandatory RDSO specification numbers explicitly quoted in Section 7 of this document.

- The system is non intrusive and can be installed on in-service locomotives without structural modifications to existing TM bearing assemblies.

- The system provides at least 6 months advance warning of bearing deterioration, enabling predictive maintenance and elimination of catastrophic failures.

- All communication protocols are open and non proprietary (Modbus TCP/IP, RS485, REST API, JSON).

- Dashboard access for Indian Railways is free of cost for the lifetime of the system.

- A 3 year warranty is provided with replacement of any defective component at no additional cost.

- Installation on each locomotive will be completed within 5 days of the train being offered.

- Manufacturing will commence only upon receipt of formal design approval from the Deputy Chief Electrical Engineer (Design and Development), Chittaranjan Locomotive Works.

Authorized Signatory

- Name: ABHAY KHATRI

- CONTACT DETAILS

  - EMAL -

  - PHONE/ MOBILE -

- Designation: PRESIDENT

- Company: BCL SECURE

- Date:  OCT 2026

- Seal: (TO BE STAMPED)